Full-spectrum LED (Light Emitting Diode) Product

  - GENERAL

- The product shall be a Fiilex P3L manufactured by DiCon Fiberoptics Inc.

- The product shall be a high-intensity LED illuminator utilizing a Dense Matrix 3D LED Array System comprised of at least 7 different LED chip colors

- The product shall comply with the USITT DMX-512 standard.

- The product shall comply with the current PLASA ANSI E1.20- 2010 remote device management (RDM) standard.

- Fiilex, a DiCon FiberOptics Inc. brand, shall provide all LED products to ensure color consistency.

B.    Each LED fixture shall be tested and optimized for optimal photometric performance.

  - PHYSICAL

- The dimensions of the fixture shall be 9.4” W x 9.5 ”H x 15.5” D (23.8 x 24.1 x 39.3 cm) and weigh approximately 7.3 lbs (3.3 kg). The following shall be provided:

  - Fiilex P3L fixture, consisting of

- P3 COLOR Head & Yoke Unit

- 90W, 24V, Track Mount PSU

- P3 DMX Adapter TRRS to XLR5

- AC Cable 6ft True1 to Edison/US

- Track Mount Adapter

- One (1) Leko Lens (choose from 19°, 26°, 36°, or 50°)

- The housing shall have a black or white finish.

- The housing material shall be PC (polycarbonate) + ABS (acrylonitrile-butadiene-styrene).

- The Gobo Size shall be Size M (Diameter 66mm), the max image shall be 48mm in diameter, and the max thickness shall be 1mm.

- Mounting Type shall have Track or Clamp selection

- Product Cooling and Electronics will be internal systems.

- Optional Accessories

- P3L Leko Lens Tube 19deg (FLXP3L-LEKO19-BK/FLXP3L-LEKO19-WH)

- P3L Leko Lens Tube 26deg (FLXP3L-LEKO26-BK/FLXP3L-LEKO26-WH)

- P3L Leko Lens Tube 36deg (FLXP3L-LEKO36-BK/FLXP3L-LEKO36-WH)

- P3L Leko Lens Tube 50deg (FLXP3L-LEKO50-BK/FLXP3L-LEKO50-WH)

# P3L Leko Gel Holder (FLXP3L-GELHOLDER-BK/FLXP3L-GELHOLDER-WH)

- P3L Leko Gobo Holder (FLXP3L-GOBOHOLDER)

- P3L Leko Gobo Set of 4 (FLXP3L-GOBOSET)

- P3L Leko Iris (FLXP3L-IRIS)

  - Track Segments

- The product shall be installed on a surface-mounted track - One Track

- Track input voltage shall be 120V

- 2 circuits shall have 20 Amps max per circuit, 40 Amps max per track

- The track shall have standard length options of 4’ and 8’ and can be cut and connected by track accessories.

- The track shall have mounting points size 0.47” x 0.16” (12mm x 4mm), Slot Per 1.64” (500mm).

- The track max load is 50kg/110 lbs (per 1.5’ of track).

- The track shall have a black or white finish.

- The material shall be Aluminum / PVC (polyvinyl chloride) / PC  (polycarbonate) / Copper conductors.

- With Data (-TOX): 7 conductors (2 Data, 2 Live, 2 Neutral, 1 Ground)

- 4’ Track with data - Black (9000-UL4/B-ST2-120) / White (9000-UL4/W-ST2-120)

- 8’ Track with data - Black (9000-UL8/B-ST2-120) / White (9000-UL8/W-ST2-120)

  - ENVIRONMENTAL AND AGENCY COMPLIANCE

- The product shall meet FCC Part 15 Subpart B, tested per ANSI C63.4 for EMI compliance.

- The product shall meet CE LVD standards per BS EN IEC 60598-1 and BS EN IEC 60598-2-1.

- The product shall meet CE EMC standards, including EN 55015, EN 61000-3-2, EN 61000-3-3, and EN 61547.

- The product shall comply with UKCA marking requirements per EN 60598-1 and EN 60598-2-1.

- The product shall comply with UK EMC standards equivalent to CE, including BS EN IEC 55015, 61000-3-2, and 61000-3-3.

- The fixture shall comply with RoHS (Restriction of Hazardous Substances) and TAA (Trade Agreements Act) regulations.

- The product shall be ETL certified to UL 1574 and CSA C22.2 for track lighting.

- The product shall be rated for IP-25 and able to sustain operation at full intensity while actively being sprayed with water from all directions.

  - Thermal

- Product heat management shall be achieved through forced cooling.

- The cooling fans shall be rated for a minimum operational lifespan of 60,000 hours.

- The product shall utilize advanced thermal management systems to maintain LED life to an average of 70% intensity after 50,000 hours of use.

- The product shall operate in an ambient temperature range of 32°F (0 °C) minimum to 104°F (40 °C) maximum.

  - Electrical

- The product shall have an auto-ranging 100 V to 240 V 50/60 Hz power supply unit.

- The product shall have a maximum draw of 90W.

- DC input Voltage shall be 14- 30V.

- The product requires power from a non-dimming source.

- Products shall have dynamic thermal monitoring at multiple locations in the LED array, control board, and other electronics to prevent thermal shift of color or intensity.

- Product power input shall have current-limiting fuse protection.

- The power supply shall have power factor correction.

  - OPTICAL DATA

- The product shall contain a patented Dense Matrix LED Light Source manufactured by DiCon FiberOptics, Inc.

- The fixture shall support swappable lenses with 19-degree, 26-degree, 36-degree, and 50-degree beam angles.

- All LEDs used in the product shall be manufactured by DiCon FiberOptics, ensuring high brightness and proven quality.

- DiCon FiberOptics, Inc. shall utilize an advanced production LED binning process to maintain color consistency.

- LED emitters should be rated for a nominal 50,000-hour LED life to 70% intensity.

- All LED products (100% of each lot) shall undergo a minimum three-hour burn-in test during manufacturing.

- The LED system shall comply with all relevant patents.

  - Color

- The product shall utilize a Dense Array of 60 LED emitters enclosed under a single half-ball lens with a diameter of 22mm.

- The Color Rendering Index (CRI) shall be 95 at 3200K, 96 at 5600K, and 97 at 6500K.

- The Television Lighting Consistency Index (TLCI) shall be 90 at 3200K, 95 at 5600K, and 96 at 6500K.

- The Correlated Color Temperature (CCT) shall be dynamic, ranging between 2000° K and 10000° K.

- The Hue range will be adjustable ±1.0 Magenta Plus/Green.

  - Calibration

- Every LED Fixture should undergo a stringent calibration process (500+ points) to ensure color consistency between fixtures and accuracy across the entire CCT range and color space.

  - Intensity Measurements

- The fixture must produce a uniform light distribution without intense brightness in the center.

  - 19° beam: Narrow light pattern with 28,099 cd (candela) peak intensity, sharp falloff.

  - 26° beam: Narrow-to-medium beam with 16,684 cd (candela) peak intensity, defined center with gradual tapering.

  - 36° beam: Medium spread with 10,129 cd (candela) peak intensity, moderate falloff.

  - 50° beam: Widespread with 1,976 cd (candela) peak intensity, gentle falloff.

- 19-degree

  - Color output @3200k distance 3’ (0.9m) at 3306fc/35587lux, distance 10’ (3.0m) at 265fc/12854lux,  distance 15’ (4.6m) at 126fc/1357lux.

  - Color output @5600k distance 3’ (0.9m) at 3564fc/38359lux, distance 10’ (3.0m) at 286fc/3080lux,  distance 15’ (4.6m) at 135fc/1455lux.

  - At 3 feet (0.9 meters), the beam spreads to 0.9 feet (0.3 meters) in diameter.

  - At 10 feet (3.0 meters), the beam spreads to 3.0 feet (0.9 meters) in diameter.

  - At 15 feet (4.6 meters), the beam spreads to 4.5 feet (1.4 meters) in diameter.

- 26-degree

  - Color output @3200k distance 3’ (0.9m) at 1978fc/21292lux, distance 10’ (3.0m) at 160fc/1726lux,  distance 15’ (4.6m) at 77fc/831lux.

  - Color output @5600k distance 3’ (0.9m) at 2079fc/22379lux, distance 10’ (3.0m) at 169fc/1814lux,  distance 15’ (4.6m) at 81fc/873lux.

  - At 3 feet (0.9 meters), the beam spreads to 1.2 feet (0.4 meters) in diameter.

  - At 10 feet (3.0 meters), the beam spreads to 4.0 feet (1.2 meters) in diameter.

  - At 15 feet (4.6 meters), the beam spreads to 5.9 feet (1.8 meters) in diameter.

- 36-degree

  - Color output @3200k distance 3’ (0.9m) at 1272fc/13692lux, distance 10’ (3.0m) at 102fc/1098lux,  distance 15’ (4.6m) at 48fc/522lux.

  - Color output @5600k distance 3’ (0.9m) at 1290fc/13890lux, distance 10’ (3.0m) at 103fc/1114lux,  distance 15’ (4.6m) at 49fc/530lux.

  - At 3 feet (0.9 meters), the beam spreads to 2.1 feet (0.6 meters) in diameter.

  - At 10 feet (3.0 meters), the beam spreads to 7.1 feet (2.2 meters) in diameter.

  - At 15 feet (4.6 meters), the beam spreads to 10.8 feet (3.3 meters) in diameter.

- 50-degree

  - Color output @3200k distance 3’ (0.9m) at 378fc/4072lux, distance 10’ (3.0m) at 31fc/335lux,  distance 15’ (4.6m) at 15fc/161lux.

  - Color output @5600k distance 3’ (0.9m) at 435fc/4678lux, distance 10’ (3.0m) at 36fc/384lux,  distance 15’ (4.6m) at 17fc/185lux.

  - At 3 feet (0.9 meters), the beam spreads to 2.1 feet (0.6 meters) in diameter.

  - At 10 feet (3.0 meters), the beam spreads to 7.1 feet (2.2 meters) in diameter.

  - At 15 feet (4.6 meters), the beam spreads to 10.8 feet (3.3 meters) in diameter.

  - Dimming

- The LED system will use an analog dimming system for a flicker-free output, tested with cameras over 20,000 fps.

- The LED system shall use 16-bit nonlinear scaling techniques for high-resolution dimming.

- The product shall have a selectable dimming curve to simulate incandescent dimming curves consistent with the following:

  - Linear

  - Gamma

- The dimming curve shall be optimized for smooth dimming over longer timed fades.

- The product shall have a selectable dimming mode consistent with the following:

  - Smooth

  - Sharp

  - REQUIRED FEATURE SET

- The product shall offer 6 DMX Control Mode Options.

- Product Modes will include: CLR (All 26 Modes), EZ (All 13  EZ Modes), RGB Mode (RGB Mode Only), CCT (Tunable CCT Mode Only), 3200 (Fixed 3200K), and 5600 (Fixed 5600K).

- The product shall offer 4 Fan modes: Auto, Full Speed, Off, and Silent

- The product shall offer a noise level not to exceed 19 dBa.

- The product shall have an over-temperature protection that will disable the fixture above 107 Celsius as measured by a thermistor embedded in the LED Array.

- The product shall offer a 307 (BTUs/hour) Max.

- The product shall offer a ±1.0 Magenta/Green shift via DMX and onboard control.

- The product shall offer full RGB color over the entire CIE 1931 color space.

- The product shall offer user-selectable Color Temperature settings.

- The product shall offer user-selectable dimming settings.

- The product shall contain a direct power/data connection.

- The product shall contain two manual knobs on the back of the fixture to control all fixture parameters.

- The product yoke assembly shall include two additional mounting point options, allowing for a lower-profile installation configuration.

- All provided products will contain the above feature set.

  - Control and User Interface

- The product shall be USITT DMX 512A-compatible via in and through 5-Pin XLR DMX Connectors or TRRS Connectors.

- The product shall offer control via DMX and RDM.

- The product shall be compatible with the ANSI RDM E1.20 standard.

- Product mode, DMX address, and fixture info shall be accessible via RDM protocol for modification from a suitably equipped control console or RDM controller.

- Temperature sensors within the luminaire shall be viewable in real time on the product control panel.

- All offered products will come with the listed RDM Compatibility.

- The product shall be equipped with a 1.01” OLED display.

- The product shall be equipped with a 2 Knob user interface.

- The product shall offer stand-alone functionality, eliminating the need for a console.

- The product shall ship with full RGB capability.

- Products without stand-alone operation features described above shall not be acceptable.

-END-