Full-spectrum LED (Light Emitting Diode) Product

  - GENERAL

- The product shall be a Fiilex P3X manufactured by DiCon Fiberoptics Inc.

- The product shall be a high-intensity LED illuminator utilizing a Dense Matrix 3D LED Array System comprised of at least 7 different LED chip colors

- The product shall comply with the USITT DMX-512 standard.

- The product shall comply with the current PLASA ANSI E1.20- 2010 remote device management (RDM) standard.

- Fiilex, a DiCon FiberOptics Inc. brand, shall provide all LED products to ensure color consistency.

B.    Each LED fixture shall be tested and optimized for optimal photometric performance.

  - PHYSICAL

- The dimensions of the fixture shall be 8.9” W x 7.8 ”H x 7.2” D (22.6 x 19.8 x 18.4 cm) and weigh approximately 3.5 lbs (1.6 kg). The following shall be provided:

  - Fiilex P3X fixture, consisting of

- P3 COLOR Head & Yoke Unit

- 90W, 24V, Clamp Mount PSU

- P3 DMX Adapter TRRS to XLR5

- AC Cable 6ft True1 to Edison/US

- P3 Barndoor 4 leaf (FLXP3CLR-BD4)

- The housing shall have a black or white finish.

- The housing material shall be PC (polycarbonate) + ABS (acrylonitrile-butadiene-styrene).

- Mounting Type shall have Track or Clamp selection

- Product Cooling and Electronics will be internal systems.

- Optional Accessories

- P3 6"x8" Softbox, Barndoor-Mount (FLXP3-6x8SB)

- P3 Fresnel Lens 20deg (FLXP3CLR-FRNL20-BK/FLXP3CLR-FRNL20-WH)

- P3 Fresnel Lens 20deg Snoot (FLXP3CLR-SNOOTF20-BK/FLXP3CLR-SNOOTF20-GR)

- Honeycomb (TRA006N)

  - ENVIRONMENTAL AND AGENCY COMPLIANCE

- The product shall meet FCC Part 15 Subpart B, tested per ANSI C63.4 for EMI compliance.

- The product shall meet CE LVD standards per BS EN IEC 60598-1 and BS EN IEC 60598-2-1.

- The product shall meet CE EMC standards, including EN 55015, EN 61000-3-2, EN 61000-3-3, and EN    61547.

- The product shall comply with UKCA marking requirements per EN 60598-1 and EN 60598-2-1.

- The product shall comply with UK EMC standards equivalent to CE, including BS EN IEC 55015, 61000-3-2, and 61000-3-3.

- The fixture shall comply with RoHS (Restriction of Hazardous Substances) and TAA (Trade Agreements Act) regulations.

- The product shall be ETL certified to UL 1574 and CSA C22.2 for track lighting.

- The product shall be rated for IP-25 and able to sustain operation at full intensity while actively being sprayed with water from all directions.

  - Thermal

- Product heat management shall be achieved through forced cooling.

- The cooling fans shall be rated for a minimum operational lifespan of 60,000 hours.

- The product shall utilize advanced thermal management systems to maintain LED life to an average of 70% intensity after 50,000 hours of use.

- The product shall operate in an ambient temperature range of 32°F (0° C) minimum to 104°F (40° C) maximum.

  - Electrical

- The product shall come with an auto-ranging 100 V to 240 V 50/60 Hz power supply unit.

- The product shall have a maximum draw of 90W.

- DC input Voltage shall be 14- 30V.

- The product requires power from a non-dimming source.

- Products shall have dynamic thermal monitoring at multiple locations in the LED array, control board, and other electronics to prevent thermal shift of color or intensity.

- Product power input shall have current-limiting fuse protection.

- The power supply shall have power factor correction.

  - OPTICAL DATA

- The product shall contain a patented Dense Matrix LED Light Source manufactured by DiCon FiberOptics, Inc.

- The fixture shall have a 60-degree native beam angle, with barn doors to cut the beam.

- All LEDs used in the product shall be manufactured by DiCon FiberOptics, ensuring high brightness and proven quality.

- DiCon FiberOptics, Inc. shall utilize an advanced production LED binning process to maintain color consistency.

- LED emitters should be rated for a nominal 50,000-hour LED life to 70% intensity.

- All LED products (100% of each lot) shall undergo a minimum three-hour burn-in test during manufacturing.

- The LED system shall comply with all relevant patents.

  - Color

- The product shall utilize a Dense Array of 60 LED emitters enclosed under a single half-ball lens with a diameter of 22mm.

- The Color Rendering Index (CRI) shall be 95 at 3200K, 96 at 5600K, and 97 at 6500K.

- The Television Lighting Consistency Index (TLCI) shall be 90 at 3200K, 95 at 5600K, and 96 at 6500K.

- The Correlated Color Temperature (CCT) shall be dynamic, ranging between 2000° K and 10000° K.

- The Hue range will be adjustable ±1.0 Magenta Plus/Green.

  - Calibration

- Every LED Fixture should undergo a stringent calibration process (500+ points) to ensure color consistency between fixtures and accuracy across the entire CCT range and color space.

  - Intensity Measurements

- The fixture must produce a uniform light distribution without intense brightness in the center. When measuring light output, the center intensity should be no more than 2x the intensity measured at 30 degrees from the center.

- Color output @3200k distance 3’ (0.9m) at 345fc/3714lux, distance 10’ (3.0m) at 36fc/388lux,  distance 15’ (4.6m) at 20fc/216lux.

- Color output @5600k distance 3’ (0.9m) at 356fc/3832lux, distance 10’ (3.0m) at 37fc/399lux,  distance 15’ (4.6m) at 21fc/227lux.

- At 3 feet (0.9 meters), the beam spreads to 2.7 feet (0.8 meters) in diameter.

- At 10 feet (3.0 meters), the beam spreads to 9.3 feet (2.8 meters) in diameter.

- At 15 feet (4.6 meters), the beam spreads to 13.9 feet (4.2 meters) in diameter.

  - Dimming

- The LED system will use an analog dimming system for a flicker-free output, tested with cameras over 20,000 fps.

- The LED system shall use 16-bit nonlinear scaling techniques for high-resolution dimming.

- The product shall have a selectable dimming curve to simulate incandescent dimming curves consistent with the following:

  - Linear

  - Gamma

- The dimming curve shall be optimized for smooth dimming over longer timed fades.

- The product shall have a selectable dimming mode consistent with the following:

  - Smooth

  - Sharp

  - REQUIRED FEATURE SET

- The product shall offer 6 DMX Control Mode Options.

- Product Modes will include: CLR (All 26 Modes), EZ (All 13  EZ Modes), RGB Mode (RGB Mode Only), CCT (Tunable CCT Mode Only), 3200 (Fixed 3200K), and 5600 (Fixed 5600K).

- The product shall offer 4 Fan modes: Auto, Full Speed, Off, and Silent

- The product shall offer a noise level not to exceed 19 dBa.

- The product shall have an over-temperature protection that disables the fixture above 107 Celsius, as measured by a thermistor embedded in the LED Array.

- The product shall offer a 307 (BTUs/hour) Max.

- The product shall offer a ±1.0 Magenta/Green shift via DMX and onboard control.

- The product shall offer full RGB color over the entire CIE 1931 color space.

- The product shall offer user-selectable Color Temperature settings.

- The product shall offer user-selectable dimming settings.

- The product shall contain a direct power/data connection.

- The product shall contain two manual knobs on the back of the fixture to control all fixture parameters.

- The product yoke assembly shall include two additional mounting point options, allowing for a lower-profile installation configuration.

- All provided products will contain the above feature set.

  - Control and User Interface

- The product shall be USITT DMX 512A-compatible via in and through 5-Pin XLR DMX Connectors or TRRS Connectors.

- The product shall offer control via DMX and RDM.

- The product shall be compatible with the ANSI RDM E1.20 standard.

- Product mode, DMX address, and fixture info shall be accessible via RDM protocol for modification from a suitably equipped control console or RDM controller.

- Temperature sensors within the luminaire shall be viewable on the product control panel in real time.

- All offered products will come with the listed RDM Compatibility.

- The product shall be equipped with a 1.01” OLED display.

- The product shall be equipped with a 2 Knob user interface.

- The product shall offer stand-alone functionality, eliminating the need for a console.

- The product shall ship with full RGB capability.

- Products without stand-alone operation features described above shall not be acceptable.

-END-